# com.qualcomm.robotcore.eventloop.opmode

public abstract class OpMode extends java.lang.Object

Gamepad			gamepad1

Gamepad 1

Gamepad			gamepad2

Gamepad 2

HardwareMap			hardwareMap

Hardware Mappings

OpModeServices		internalOpModeServices

This is logically an internal field.

int				msStuckDetectInit

int				msStuckDetectInitLoop

int				msStuckDetectLoop

int				msStuckDetectStart

int				msStuckDetectStop

Telemetry			telemetry

The telemetry field contains an object in which a user may accumulate data which is to be transmitted to the driver station.

double			time

number of seconds this op mode has been running, this is updated before every call to loop.

public abstract class LinearOpMode extends OpMode

protected void			handleLoop()

void			idle()

Puts the current thread to sleep for a bit as it has nothing better to do.

void				internalPostInitLoop()

automatically update telemetry in a non-linear opmode

void				internalPostLoop()

automatically update telemetry in a non-linear opmode

boolean			isStarted()

Has the opMode been started?

boolean			isStopRequested()

Has the the stopping of the opMode been requested?

boolean			opModeIsActive()

Answer as to whether this opMode is active and the robot should continue onwards.

abstract void			runOpMode()

Override this method and place your code here.

void				sleep(long milliseconds)

Sleeps for the given amount of milliseconds, or until the thread is interrupted.

void				waitForNextHardwareCycle()

Deprecated.

The need for user code to synchronize with the loop() thread has been obviated by improvements in the modern motor and servo controller implementations. Remaining uses of this API are likely unnecessarily wasting cycles. If a simple non-zero delay is required, the sleep() method is a better choice. If one simply wants to allow other threads to run, idle() is a good choice.

void				waitForStart()

Pauses the Linear Op Mode until start has been pressed or until the current thread is interrupted.

# org.firstinspires.ftc.robotcore.external

public interface Telemetry

Object				addAction(Runnable action)

In addition to items and lines, a telemetry may also contain a list of actions.

<T> Telemetry.Item		addData(String caption, Func<T> valueProducer)

Adds an item to the end of the telemetry being built for driver station display.

Telemetry.Item			addData(String caption, Object value)

Adds an item to the end if the telemetry being built for driver station display.

<T> Telemetry.Item		addData(String caption, String format, Func<T> valueProducer)

Adds an item to the end of the telemetry being built for driver station display.

Telemetry.Item			addData(String caption, String format, Object... args)

Adds an item to the end of the telemetry being built for driver station display.

Telemetry.Line			addLine()

Creates and returns a new line in the receiver Telemetry.

Telemetry.Line			addLine(java.lang.String lineCaption)

Creates and returns a new line in the receiver Telemetry.

void				clear()

Removes all items from the receiver whose value is not to be retained.

void				clearAll()

Removes all items, lines, and actions from the receiver

String				getCaptionValueSeparator()

Returns the string which is used to separate caption from value within a Telemetry Telemetry.Item.

String				getItemSeparator()

Returns the string which is used to separate Telemetry.Items contained within a line.

int				getMsTransmissionInterval()

Returns the minimum interval between Telemetry transmissions from the robot controller to the driver station

boolean			isAutoClear()

Answers whether clear() is automatically called after each call to update().

Telemetry.Log			log()

Returns the log of this Telemetry to which log entries may be appended.

boolean			removeAction(java.lang.Object token)

Removes a previously added action from the receiver.

boolean			removeItem(Telemetry.Item item)

Removes an item from the receiver telemetry, if present.

boolean			removeLine(Telemetry.Line line)

Removes a line from the receiver telemetry, if present.

void				setAutoClear(boolean autoClear)

Sets whether clear() is automatically called after each call to update().

void				setCaptionValueSeparator(java.lang.String captionValueSeparator)

void				setItemSeparator(java.lang.String itemSeparator)

void				setMsTransmissionInterval(int msTransmissionInterval)

Sets the minimum interval between Telemetry transmissions from the robot controller to the driver station.

boolean			update()

Sends the receiver Telemetry to the driver station if more than the transmission interval has elapsed since the last transmission, or schedules the transmission of the receiver should no subsequent Telemetry state be scheduled for transmission before the transmission interval expires.

# org.firstinspires.ftc.robotcore.external.navigation

## public enum AngleUnit

## extends java.lang.Enum<AngleUnit>

DEGREES

RADIANS

An AngleUnit represents angles in different units of measure and provides utility methods to convert across units. AngleUnit does not maintain angle information internally, but only helps organize and use angle measures that may be maintained separately across various contexts.

Angles can be distinguished along (at least) two axes:

the fundamental unit (radians vs degrees)

whether the angular quantity is normalized or not to the range of [-180,+180) degrees

Normalized angles are of most utility when dealing with physical angles, as normalization removes ambiguity of representation. In particular, two angles can be compared for equality by subtracting them, normalizing, and testing whether the absolute value of the result is smaller than some tolerance threshold. This approach neatly handles all cases of cyclical wrapping without unexpected discontinuities.

Unnormalized angles can be handy when the angular quantity is not a physical angle but some related quantity such as an angular velocity or acceleration, where the quantity in question lacks the 360-degree cyclical equivalence of a physical angle.

AngleUnit expresses normalized angles, while UnnormalizedAngleUnit expresses unnormalized ones

# com.qualcomm.robotcore.hardware

Interface HardwareDevice

Static class HardwareDevice.Manufacturer

void					close()

Closes this device

String					getConnectionInfo()

Get connection information about this device in a human readable format

String					getDeviceName()

Returns a string suitable for display to the user as to the type of device.

HardwareDevice.Manufacturer		getManufacturer()

Returns an indication of the manufacturer of this device.

int					getVersion()

Version

void					resetDeviceConfigurationForOpMode()

Resets the device's configuration to that which is expected at the beginning of an OpMode.

Interface DcMotorSimple

Public static enum 		DcMotorSimple.Direction

DcMotors can be configured to internally reverse the values to which, e.g., their motor power is set. This makes it easy to have drive train motors on two sides of a robot: during initialization, one would be set at at forward, the other at reverse, and the difference between the two in that respect could be thereafter ignored.

At the start of an OpMode, motors are guaranteed to be in the forward direction.

FORWARD

REVERSE

DcMotorSimple.Direction		getDirection()

Returns the current logical direction in which this motor is set as operating.

double					getPower()

Returns the current configured power level of the motor.

void					setDirection(DcMotorSimple.Direction direction)

Sets the logical direction in which this motor operates.

void					setPower(double power)

Sets the power level of the motor, expressed as a fraction of the maximum possible power / speed supported according to the run mode in which the motor is operating.

Public interface DcMotor

Public static enum 		DcMotor.RunMode

The run mode of a motor DcMotor.RunMode controls how the motor interprets the it's parameter settings passed through power- and encoder-related methods.

RUN_TO_POSITION

The motor is to attempt to rotate in whatever direction is necessary to cause the encoder reading to advance or retreat from its current setting to the setting which has been provided through the setTargetPosition() method. An encoder must be affixed to this motor in order to use this mode. This is a PID mode. It adds an additional P loop that makes the motor run to and stay on a position. Even at full power, the motor will not run past it’s target.

RUN_USING_ENCODER

The motor is to do its best to run at targeted velocity. It uses a PID loop inside the REV hub to stabilize the speed of a motor. The voltage is fluctuated such that the motor runs a certain amount of encoder ticks per second. Setting a velocity from RUN_WITHOUT_ENCODER mode will automatically switch the motor to RUN_USING_ENCODER mode. You should pick a velocity that the motor will be capable of reaching even with a full load and a low battery.

RUN_WITHOUT_ENCODER

The motor is simply to run at whatever velocity is achieved by apply a particular power level to the motor. It feeds a raw voltage into the motor. The encoder is still active, but it doesn’t effect the speed of the motor.

STOP_AND_RESET_ENCODER

The motor is to set the current encoder position to zero. In contrast to RUN_TO_POSITION, the motor is not rotated in order to achieve this; rather, the current rotational position of the motor is simply reinterpreted as the new zero value. However, as a side effect of placing a motor in this mode, power is removed from the motor, causing it to stop, though it is unspecified whether the motor enters brake or float mode. Further, it should be noted that setting a motor to STOP_AND_RESET_ENCODER may or may not be a transient state: motors connected to some motor controllers will remain in this mode until explicitly transitioned to a different one, while motors connected to other motor controllers will automatically transition to a different mode after the reset of the encoder is complete.

public static enum 		DcMotor.ZeroPowerBehavior

BRAKE

The motor stops and then brakes, actively resisting any external force which attempts to turn the motor.

FLOAT

The motor stops and then floats: an external force attempting to turn the motor is not met with active resistance.

UNKNOWN

The behavior of the motor when zero power is applied is not currently known.

DcMotorController			getController()

Returns the underlying motor controller on which this motor is situated.

int					getCurrentPosition()

Returns the current reading of the encoder for this motor.

DcMotor.RunMode			getMode()

Returns the current run mode for this motor

MotorConfigurationType		getMotorType()

Returns the assigned type for this motor.

int					getPortNumber()

Returns the port number on the underlying motor controller on which this motor is situated.

boolean				getPowerFloat()

Returns whether the motor is currently in a float power level.

int					getTargetPosition()

Returns the current target encoder position for this motor.

DcMotor.ZeroPowerBehavior		getZeroPowerBehavior()

Returns the current behavior of the motor were a power level of zero to be applied.

boolean				isBusy()

Returns true if the motor is currently advancing or retreating to a target position.

void					setMode(DcMotor.RunMode mode)

Sets the current run mode for this motor

- RUN_TO_POSITION

- RUN_USING_ENCODER

- RUN_WITHOUT_ENCODER

- STOP_AND_RESET_ENCODER

void					setMotorType(MotorConfigurationType motorType)

Sets the assigned type of this motor.

void					setTargetPosition(int position)

Sets the desired encoder target position to which the motor should advance or retreat and then actively hold thereat.

void					setZeroPowerBehavior(DcMotor.ZeroPowerBehavior )

Sets the behavior of the motor when a power level of zero is applied.

public class PIDFCoefficients extends Object

PIDFCoefficients conveys a set of configuration parameters for a PIDF algorithm, a PID algorithm which includes an additional feed-forward term. Optionally, coefficients for the original Expansion Hub "Legacy PID" algorithm may also be indicated.

MotorControlAlgorithm	algorithm

double				d

double				f

double				i

double				p

public enum MotorControlAlgorithm extends Enum<MotorControlAlgorithm>

LegacyPID

Deprecated.  Switch to PIDF instead

PIDF

Unknown

public interface DcMotorEx

PIDFCoefficients		getPIDFCoefficients(DcMotor.RunMode mode)

Returns the PIDF control coefficients used when running in the indicated mode on this motor.

int				getTargetPositionTolerance()

Returns the current target positioning tolerance of this motor

double				getVelocity()

Returns the current velocity of the motor, in ticks per second

double				getVelocity(AngleUnit unit)

Returns the current velocity of the motor, in angular units per second

boolean			isMotorEnabled()

Returns whether this motor is energized

void				setMotorDisable()

Individually de-energizes this particular motor

void				setMotorEnable()

Individually energizes this particular motor

void				setPIDFCoefficients(DcMotor.RunMode, PIDFCoefficients)

setPIDFCoefficients(DcMotor.RunMode, PIDFCoefficients) is a superset enhancement to setPIDCoefficients(DcMotor.RunMode, PIDCoefficients).

void				setPositionPIDFCoefficients(double p)

A shorthand for setting the PIDF coefficients for the RUN_TO_POSITION mode.

void				setTargetPositionTolerance(int tolerance)

Sets the target positioning tolerance of this motor

void				setVelocity(double angularRate)

Sets the velocity of the motor. It seems to do this by setting a certain power level then rely on a control loop to reach that velocity. It seems to release the isBusy() flag before it reaches the target velocity. The maximum velocity seems to be 360 degrees / second (1Hz). setTargetPositionTolerance() does not seem to make a difference in the algorithm here.

void				setVelocity(double angularRate, AngleUnit unit)

Sets the velocity of the motor

void				setVelocityPIDFCoefficients(

double p,

double i,

double d,

double f)

A shorthand for setting the PIDF coefficients for the RUN_USING_ENCODER mode.

# Org.firstinspires.ftc.teamcode

PinpointDrive extends MecanumDrive

# com.acmerobotics.roadrunner.ftc

class GoBildaPinpointDriver

setEncoderResolution(double)

setOffsets(double, double)

setEncoderDirections(GoBildaPinpointDriver.EncoderDirection, GoBildaPinpointDriver.EncoderDirection)

resetPosAndIMU()

will set position to 0,0,0 and recalibrate the IMU

setPosition(

Suggested Improvements

- Rely on hardware objects to own hardware parameters

# IMU

Makek sure you calibrate the IMU before games. This needs to happen when the robot is stationary. The IMU will automatically calibrate when first powered on. But recalibrating before running the robot is a good idea to ensure that the calibration is good